HIGH-DENSITY ELECTROCARDIOGRAPHIC

IMAGING SYSTEM

Technical Proposal & Investment Memorandum

Version 6.0

29 December 2025

Funding Request: £15,000,000

CRITICAL READER NOTICE This document describes a medical device that does not yet exist. No prototype has been built. No clinical data exists. All performance figures are engineering targets requiring validation. This proposal requests seed funding to validate core technical assumptions before Series A.

# Table of Contents

Table of Contents	2

Executive Summary	4

The Problem	4

The Core Insight	4

The Solution	4

The Ask	4

Evidence Classification	4

1. Technical Specification	5

1.1 System Architecture	5

1.2 CT-Free Geometry Estimation	5

1.2.1 Technical Approach	5

1.2.2 Literature Precedent	5

1.2.3 CT Training Data Acquisition	5

1.3 Electrode Contact Probability	6

1.3.1 Revised Model	6

1.3.2 Cluster Failure Analysis	6

2. Optical Multiplexing System	7

2.1 Component-Level Timing Budget	7

2.2 Key Personnel — Optical Engineering	7

3. Liability Framework — Pre-Hospital Use	8

3.1 Device Classification	8

3.2 Liability Matrix	8

3.3 Legal Safe Harbours	8

3.4 Regulatory Safe Harbour	8

4. Demographic Subgroup Validation	9

4.1 Training Data Demographic Gap Analysis	9

4.2 Stratified Validation Protocol	9

4.3 Bias Detection Protocol	9

5. Worst-Case Scenario Analysis	10

5.1 Algorithm Bias Death Scenario	10

5.1.1 Cascade Analysis	10

5.1.2 Design Principle: Fail Safe, Not Fail Silent	10

5.2 Financial Worst Case — Liability Exposure	10

6. Probability Weight Derivation	11

6.1 Named Advisors	11

6.2 Methodology	11

6.3 Scenario Probabilities	11

6.4 Sensitivity Analysis	11

7. End User Deployment & Cognitive Load	12

7.1 Deployment Time — Evidence Recalibration	12

7.2 Alert States — Training Requirements	12

7.3 NASA-TLX Validation Plan	12

8. Financial Model	13

8.1 Funding Allocation — Phase 1 (£2.8M)	13

8.2 NPV Model Summary	13

8.3 Market Validation	13

9. Security Architecture	14

9.1 Build Pipeline Security	14

9.2 Cloud Security — Threat Model	14

10. Strategic Pivot vs Graceful Degradation	15

10.1 Honest Assessment	15

10.2 CT-Based Pivot — Competitive Analysis	15

10.3 Investor Implications	15

11. Development Gates	16

11.1 Phase 1 Decision Points	16

11.2 Termination Triggers	16

12. Regulatory Compliance	17

12.1 EU AI Act — High-Risk Classification	17

12.2 MDR Article 61 — Clinical Evaluation	17

Appendix A: Glossary	18

Appendix B: References	19

Appendix C: Document Control	20

# Executive Summary

## The Problem

Current pre-hospital emergency cardiac care lacks portable, high-resolution 3D cardiac electrical mapping capability. Electrophysiology laboratory precision remains confined to hospital settings, creating a critical gap in early intervention for acute cardiac events. Paramedics and first responders must rely on standard 12-lead ECG interpretation, which provides limited spatial resolution for complex arrhythmia localisation and STEMI detection in atypical presentations.

## The Core Insight

A 700-electrode high-density array combined with statistical shape modelling can potentially eliminate the requirement for CT imaging in cardiac geometry estimation. If validated, this would enable deployment of electrophysiology-grade mapping in pre-hospital settings for the first time—a capability no existing technology provides.

## The Solution

The HD-ECGI system comprises a rapid-deployment vest with 700 surface electrodes, optical multiplexing for signal aggregation, and edge/cloud processing for real-time cardiac mapping. The system provides actionable clinical decision support to paramedics, displaying confidence-weighted outputs that integrate with existing care pathways.

## The Ask

We seek £15,000,000 in seed funding across three phases to validate the core technical hypothesis, build a functional prototype, and conduct initial clinical validation. Phase 1 (£2.8M) focuses on proof-of-concept validation with explicit go/no-go gates. The investment thesis depends on CT-free algorithm success; failure triggers pivot or wind-down with controlled loss.

## Evidence Classification

All claims in this document are explicitly tagged to ensure transparency regarding evidence quality:

- [DEMONSTRATED] — Validated by cited peer-reviewed literature or completed work product

- [TARGET] — Engineering specification requiring validation; development risk acknowledged

- [PROJECTED] — Financial or market estimate based on stated assumptions

- [HYPOTHESIS] — Proposition to be tested; not evidence for investment

# 1. Technical Specification

## 1.1 System Architecture

[TARGET] The HD-ECGI system comprises 700 surface electrodes in a rapid-deployment vest configuration, optical multiplexing for signal aggregation, and edge/cloud processing for real-time cardiac mapping.

| Parameter | Specification | Evidence Level |
| --- | --- | --- |
| Electrode Count | 700 | [TARGET] Design specification |
| Sampling Rate | 4000 Hz/channel | [DEMONSTRATED] Potyagaylo 2019 |
| ADC Resolution | 24-bit | [DEMONSTRATED] ADS1299 |
| CT-free Position Accuracy | ±20mm | [TARGET] vs ±18mm Schulze 2019 |
| Deployment Time | <120 seconds | [TARGET] Requires validation |
| Weight | <5 kg | [TARGET] Mass budget completed |
| Battery Life | 4 hours continuous | [TARGET] 15W power budget |

## 1.2 CT-Free Geometry Estimation

The CT-free algorithm is a novel research problem. We do not claim to have solved it. This section describes our approach and evidence base.

### 1.2.1 Technical Approach

[HYPOTHESIS] Heart position can be estimated to ±20mm accuracy without CT using Statistical Shape Models (SSM) constrained by surface electrode positions and body habitus parameters.

The methodology comprises four elements: Statistical Shape Model reducing heart position to approximately 15 principal components derived from N≥500 training CT scans; body habitus constraints using height, weight, chest circumference, and BMI as Bayesian priors; electrode position sensing through 3D positions inferred from inter-electrode impedance patterns; and validation against held-out CT cohort comparison with pre-specified accuracy threshold.

### 1.2.2 Literature Precedent

[DEMONSTRATED] Schulze et al. (2019) achieved ±18mm accuracy using SSM with 12-lead electrode positions. This represents a single study under controlled laboratory conditions with N=47 participants, majority Caucasian population.

Limitations of precedent include: extrapolation from 12 electrodes to 700 is conjectural; laboratory conditions versus pre-hospital environment; unknown demographic generalisability; and no independent replication cited. Our hypothesis that 700 electrodes provide additional spatial constraints improving accuracy is the proposition to be tested, not evidence for investment.

### 1.2.3 CT Training Data Acquisition

Critical clarification: UK Biobank provides cardiac MRI, not CT. MESA provides MRI. This discrepancy has been resolved with the following CT data acquisition strategy:

| Source | Description | Status |
| --- | --- | --- |
| NHS PACS Access | Cardiac CT angiography scans via NHS Trust radiology under research data sharing agreement | Letter of Intent from UHB Radiology |
| SCOT-HEART Trial | 4,146 participants with CCTA; Edinburgh dataset | Application submitted; decision expected Q1 2025 |
| Commercial Dataset | Arterys/Tempus cardiac CT repository | ~£15K for N=500 anonymised scans |
| Prospective Collection | New CCTA patients consented during Phase 2 | Budgeted: £50K for N=200 paired scans |

## 1.3 Electrode Contact Probability

Adjacent electrodes share failure modes including skin contact quality, hair interference, and movement. The binomial independence assumption is structurally invalid and has been replaced with a spatial correlation model.

### 1.3.1 Revised Model

The model parameters are: base per-electrode contact probability of 70% [TARGET—conservative estimate pending validation]; spatial correlation coefficient ρ = 0.3 for adjacent electrodes (electrodes within 2cm share 30% of failure variance); cluster failure model where if one electrode fails, adjacent electrodes have P(fail) = 0.3 + 0.7 × 0.3 = 51%; Monte Carlo simulation (N=10,000) yielding expected good contacts of 483 ± 47 versus 490 under independence assumption.

### 1.3.2 Cluster Failure Analysis

| Scenario | P(scenario) | Good Electrodes | System Impact |
| --- | --- | --- | --- |
| Normal | 75% | 450-520 | Full capability |
| Regional cluster (hair) | 15% | 380-450 | Degraded resolution in one region |
| Multiple clusters | 8% | 300-380 | Basic arrhythmia only |
| Catastrophic (sweating) | 2% | <300 | Device declares UNRELIABLE |

Mitigation: Electrode density provides redundancy. Critical diagnostic regions (anterior precordium) have 3× electrode density.

# 2. Optical Multiplexing System

## 2.1 Component-Level Timing Budget

[TARGET] 8:1 optical multiplexing at 4000 Hz with <50μs inter-channel skew.

| Component | Time (μs) | Source | Status |
| --- | --- | --- | --- |
| LED driver switching | 2.1 | LT3092 datasheet | Validated |
| Photodiode response | 0.8 | OPT101 datasheet | Validated |
| Transimpedance amplifier settling | 5.2 | Design spec | Requires PoC |
| ADC acquisition | 4.0 | ADS1299 datasheet | Validated |
| FPGA synchronisation | 1.5 | Design spec | Requires PoC |
| Digital bus latency | 3.4 | SPI @ 20MHz | Validated |
| TOTAL | 17.0 | — | 66% margin to 50μs |

Three components require proof-of-concept validation. Budget total: 17μs versus 50μs requirement = 66% margin.

## 2.2 Key Personnel — Optical Engineering

| Role | Candidate | Status |
| --- | --- | --- |
| Primary | Dr James Chen, PhD Imperial, 8y Lumentum | Informal interest; 60-day window |
| Backup 1 | Dr Richard Harris, OptoMed Consulting | Available at £850/day; confirmed |
| Backup 2 | Southampton Optoelectronics Lab (Prof. Sarah Jenkins) | 6-month ethics/IP lead time |
| Backup 3 | Dr Mei Wong, ex-II-VI Incorporated | Available Q2 2025 |

Contingency budget if primary candidate unavailable: 6-month engagement at £850/day × 22 days/month = £112,200. Contingency reserve of £120K allocated in Phase 1 budget specifically for this scenario.

# 3. Liability Framework — Pre-Hospital Use

## 3.1 Device Classification

[TARGET] Clinical Decision Support (CDS) under EU MDR Article 2(1) and FDA 21 CFR 820. The device provides information to support clinical decisions; it does not make autonomous diagnostic or treatment decisions.

Regulatory classification: EU MDR Class IIb (active diagnostic device influencing patient management); FDA Class II via De Novo pathway (novel device, moderate risk); UK MHRA UKCA Class IIb (aligned with EU).

## 3.2 Liability Matrix

| Scenario | Device Output | Clinical Decision | Liability |
| --- | --- | --- | --- |
| True positive | STEMI detected | Paramedic routes to cath lab | No liability (correct) |
| False negative — device error | No STEMI (LOW confidence) | Routes to A&E per protocol | Manufacturer (defect) |
| False negative — user error | INSUFFICIENT CONTACT alert ignored | Routes to A&E | User (training failure) |
| False positive | STEMI (HIGH confidence) | Unnecessary cath lab activation | Shared (acceptable FP rate) |
| Demographic bias death | Misclassification due to underrepresented population | Patient dies | Manufacturer (bias as defect) |

## 3.3 Legal Safe Harbours

Four mechanisms provide liability protection: Labelling with clear 'Decision Support—Clinical Judgement Required' designation and on-screen display stating 'This is not a diagnosis. Clinical assessment is required.' Confidence disclosure where all outputs include confidence score and LOW confidence mandates standard of care pathway. Training certification requiring operators to complete certified training programme with liability shifting to employer if uncertified personnel operate device. Indemnification with product liability insurance minimum £10M per occurrence and indemnification clause in EMS procurement contracts.

## 3.4 Regulatory Safe Harbour

Critical design principle: When algorithm confidence is below threshold, device outputs ALERT requiring standard of care pathway. Confidence threshold of 85% [TARGET—to be validated in clinical trials]. Below threshold displays 'LOW CONFIDENCE—STANDARD ASSESSMENT REQUIRED'. Regulatory intent demonstrates that device fails safe; uncertain cases escalate to human judgement.

# 4. Demographic Subgroup Validation

## 4.1 Training Data Demographic Gap Analysis

| Population | UK Population | Training Data | Gap |
| --- | --- | --- | --- |
| White British | 80% | 94% | Overrepresented |
| South Asian | 8% | 2% | CRITICAL GAP — 4× under |
| Afro-Caribbean | 3% | 1.5% | SIGNIFICANT GAP — 2× under |
| BMI > 40 | 4% | <1% | CRITICAL GAP — 4× under |
| Female | 51% | 45% | Moderate gap |

## 4.2 Stratified Validation Protocol

Minimum validation sample sizes per subgroup (statistical power 80%, alpha 0.05, effect size 0.3):

| Subgroup | Minimum N | Recruitment Strategy |
| --- | --- | --- |
| South Asian | N ≥ 120 | Partnership with Barts Heart Centre (40% South Asian catchment) |
| Afro-Caribbean | N ≥ 80 | Partnership with King's College Hospital |
| BMI > 40 | N ≥ 100 | Bariatric surgery waitlist recruitment via NHS Trusts |
| Female | N ≥ 200 | 50% enrollment target across all sites |

## 4.3 Bias Detection Protocol

Prospective audit with sensitivity/specificity calculated separately for each subgroup. Disparity threshold triggers algorithm retraining if subgroup sensitivity falls below 90% of majority population sensitivity. Per-subgroup confidence score calibration using Platt scaling. Labelling requirement: if validation N is below minimum, device label states 'Performance not validated for [subgroup]'.

# 5. Worst-Case Scenario Analysis

## 5.1 Algorithm Bias Death Scenario

Scenario: Product ships. Algorithm misclassifies STEMI as stable in South Asian male. Paramedic follows device output. Patient dies in ambulance. Investigation reveals training data lacked demographic coverage.

### 5.1.1 Cascade Analysis

| Step | Event | Mitigation |
| --- | --- | --- |
| 1 | Biased training data | Stratified validation protocol with minimum N per subgroup |
| 2 | Algorithm confidence high despite misclassification | Per-subgroup confidence calibration; uncalibrated subgroup → automatic LOW confidence |
| 3 | Paramedic trusts device output | Training emphasises device as support tool; mandatory clinical assessment documentation |
| 4 | Standard of care not followed | Device logs all clinical decisions; audit trail demonstrates guideline adherence |
| 5 | Patient death | Incident response: immediate pause, root cause analysis, regulatory notification within 24 hours |
| 6 | Coroner's inquest | Full disclosure policy; audit trail available; design documentation demonstrating good faith |
| 7 | Criminal negligence referral | Legal safe harbours; documented MDR essential requirements compliance |

### 5.1.2 Design Principle: Fail Safe, Not Fail Silent

Unknown population: If patient demographics unavailable or outside validated range, output LOW CONFIDENCE. Atypical presentation: ECG patterns outside training distribution trigger LOW CONFIDENCE. Edge cases: Rather than guess, device defaults to standard of care escalation.

## 5.2 Financial Worst Case — Liability Exposure

| Scenario | Exposure |
| --- | --- |
| Single patient death (civil) | £1-5M per claim |
| Class action (systematic bias) | £50-200M |
| Product recall | £5-20M (depends on deployment scale) |
| Regulatory fine (MDR violation) | Up to €20M or 4% global turnover |
| Reputational damage | Company termination |

Mitigation: £10M product liability insurance; D&O insurance for directors; documented good faith compliance.

# 6. Probability Weight Derivation

## 6.1 Named Advisors

The following advisors provided probability estimates under documented methodology:

| Name | Credentials | Conflict Disclosure | Weight |
| --- | --- | --- | --- |
| Dr Helen Morrison | Former VP R&D, Medtronic CRM; 25 years medical devices | No equity; £5K honorarium | 2× |
| Prof. David Chen | Imperial College; 40+ publications ECG signal processing | Potential scientific advisory role | 1.5× |
| Sarah Thompson | Partner, MedTech Ventures; 15 cardiac device investments | May invest; recused from Full Success estimate | 1× |
| Dr James Patel | Consultant Cardiologist, UHB; EP specialist | No financial interest | 1.5× |
| Mark Williams | Former Programme Director, NHS Digital; regulatory specialist | No financial interest | 1× |

## 6.2 Methodology

Base rates derived from medical device phase success rates per FDA/industry data (Makower et al., 2010). Project adjustments: each advisor provided adjustment factors for novel algorithm risk (+), experienced team (−), validated market (−), and pre-prototype stage (+). Elicitation via independent structured questionnaire with no group discussion to avoid anchoring. Aggregation by weighted average based on domain expertise relevance.

## 6.3 Scenario Probabilities

| Scenario | Base Rate | Adjusted | Final | Range |
| --- | --- | --- | --- | --- |
| Full Success | 8% | +7% | 15% | 10-22% |
| Partial (Hospital only) | 20% | +5% | 25% | 18-32% |
| Partial (CT-based) | 25% | +5% | 30% | 22-38% |
| Pivot (components) | 12% | -2% | 10% | 5-15% |
| Total Failure | 35% | -15% | 20% | 12-28% |

## 6.4 Sensitivity Analysis

Break-even probability: Full success ≥8% yields non-negative expected NPV at 10% discount rate.

| Full Success Probability | Expected NPV | Assessment |
| --- | --- | --- |
| 5% (hostile case) | -£4.1M | Investment loses money |
| 8% (break-even) | £0 | Risk-neutral threshold |
| 15% (base case) | +£1.3M | Positive expected value |
| 22% (optimistic) | +£3.8M | Strong risk-adjusted return |

# 7. End User Deployment & Cognitive Load

## 7.1 Deployment Time — Evidence Recalibration

Deployment time target: <120 seconds [TARGET]. This requires human factors validation.

Human factors validation study (budgeted in Phase 2): N=30 paramedics across 3 NHS Ambulance Trusts; simulated deployment scenarios including moving ambulance, patient combativeness, low light; primary endpoint is median deployment time with 90th percentile; Go/No-Go criterion: if 90th percentile exceeds 180 seconds, redesign required before Phase 3.

## 7.2 Alert States — Training Requirements

| Alert State | Display | Action Required | Training Time |
| --- | --- | --- | --- |
| READY | Green | Proceed with analysis | 5 min |
| PROVISIONAL | Amber — contact <80% | Result valid but may miss localised abnormality; document in PCR | 15 min |
| UNRELIABLE | Red — contact <50% | Do not use output; proceed with standard 12-lead | 10 min |
| CLOUD UNAVAILABLE | Grey — no network | Local analysis only (basic arrhythmia); no 3D mapping; clinical assessment required | 15 min |

Total training time for alert response: 45 minutes. Comparable to AED training (30-60 minutes).

## 7.3 NASA-TLX Validation Plan

[HYPOTHESIS] Net cognitive load decrease when HD-ECGI replaces 12-lead pattern recognition task.

Validation protocol: Randomised crossover study with each paramedic serving as own control. Conditions: (A) Standard 12-lead interpretation, (B) HD-ECGI with binary output. Measure: NASA-TLX (6 subscales) administered immediately post-scenario. Sample size: N=40 (power 80% to detect 15% reduction in overall workload). Timeline: Phase 2, Q3 2026. Note: Until validation complete, cognitive load reduction remains a hypothesis, not evidence for the investment case.

# 8. Financial Model

## 8.1 Funding Allocation — Phase 1 (£2.8M)

| Category | Budget | Notes |
| --- | --- | --- |
| 64-channel benchtop PoC | £45,000 | Go/No-Go at Month 3 |
| Optical engineer (18 months) | £180,000 | £100K salary + benefits |
| Optical engineer CONTINGENCY | £120,000 | Consultant fallback |
| Algorithm development (team of 3) | £450,000 | ML engineer + 2 researchers |
| CT data acquisition | £65,000 | SCOT-HEART + commercial |
| Market validation study (N≥50) | £85,000 | Structured WTP analysis |
| Regulatory consultancy | £120,000 | MDR/FDA pathway prep |
| IP & legal | £80,000 | Patent filings |
| Operations & facilities | £200,000 | Lab space, equipment |
| Programme management | £150,000 | 1 FTE + admin support |
| Contingency (15%) | £305,000 | Unallocated reserve |
| TOTAL PHASE 1 | £2,800,000 | 18 months |

## 8.2 NPV Model Summary

Discount rate: 10% (medical device WACC benchmark per Makower et al., 2010)

| Scenario | NPV | Probability | Contribution |
| --- | --- | --- | --- |
| Full Success | £42M | 15% | +£6.3M |
| Hospital Only | £18M | 25% | +£4.5M |
| CT-Based Pivot | £5M | 30% | +£1.5M |
| Component Sale | -£8M | 10% | -£0.8M |
| Total Failure | -£15M | 20% | -£3.0M |
| EXPECTED NPV | — | 100% | +£1.3M |

## 8.3 Market Validation

Current market validation of N=12 interviews is acknowledged as insufficient for investment-grade diligence. Phase 1 includes structured market validation: sample N≥50 across EMS directors, EP cardiologists, hospital procurement; methodology using Van Westendorp price sensitivity analysis and conjoint for feature prioritisation; deliverable of willingness-to-pay distribution with confidence intervals; timeline Month 6-12 of Phase 1 with results informing Phase 2 business case.

# 9. Security Architecture

## 9.1 Build Pipeline Security

| Control | Implementation |
| --- | --- |
| Code Signing Ceremony | Dual-person control; signing keys held on HSM (Yubikey FIPS); ceremony witnessed and logged |
| Reproducible Builds | Hermetic build environment (Bazel); binary reproducibility verification by independent party |
| Supply Chain Attestation | SLSA Level 3 compliance; in-toto attestations for all build steps |
| Build Server Integrity | Ephemeral build agents; no persistent state; agent integrity verified before each build |
| Audit Trail | Immutable logs (append-only); Git signed commits; SBOM generated for each release |

Release authorization requires dual control (CTO + Security Lead). Pre-release checklist includes security scan, regression tests, and manual review. Post-release verification by independent validation of signature chain.

## 9.2 Cloud Security — Threat Model

| Threat | Vector | Mitigation |
| --- | --- | --- |
| Data exfiltration | Compromised cloud instance | Encryption at rest (AES-256); no patient identifiers transmitted |
| Model poisoning | Adversarial training data | Data provenance tracking; model version control; rollback capability |
| Denial of service | Cloud unavailability | Local fallback processing; device operates offline with reduced capability |
| Man-in-the-middle | Network interception | Certificate pinning; mutual TLS authentication |
| Re-identification | ECG biometric uniqueness | No ECG stored with demographics; time-limited processing tokens |

Penetration testing: Scope covers device firmware, cloud infrastructure, and mobile app. Annual third-party assessment plus continuous automated scanning. Budget: £50K/year (Phase 2 onwards).

# 10. Strategic Pivot vs Graceful Degradation

## 10.1 Honest Assessment

If CT-free algorithm fails, this constitutes a strategic pivot to a different product serving a different market with different competitive dynamics—not graceful degradation.

| Scenario | Product | Market Position |
| --- | --- | --- |
| Full Success | Pre-hospital HD-ECGI | Blue ocean — no direct competitors |
| CT-Free Fails | Hospital CT-based HD-ECGI | Red ocean — CardioInsight incumbent |

## 10.2 CT-Based Pivot — Competitive Analysis

If pivot to CT-based hospital system is required: CardioInsight has 10+ years clinical validation with installed base in approximately 200 EP labs. Our differentiator: 700 electrodes versus 252 (density advantage) plus open-source (cost advantage). Realistic market capture: 5-10% of EP lab market over 5 years versus 30%+ for pre-hospital. NPV in this scenario: £5M versus £42M for full success.

This is not 'graceful degradation'—it is a different business with lower expected returns competing against an entrenched incumbent.

## 10.3 Investor Implications

The investment thesis depends on CT-free success. If CT-free fails, options are: (1) Pivot to CT-based and accept lower returns; (2) Sell components/IP and return capital; (3) Wind down with controlled loss. Decision point: End of Phase 1B (Month 18). If CT-free algorithm does not achieve ±30mm accuracy on validation set, pivot decision required.

# 11. Development Gates

## 11.1 Phase 1 Decision Points

| Month | Gate | Criterion | Action if Fail |
| --- | --- | --- | --- |
| 3 | Benchtop PoC | 64-ch optical mux achieves <50μs skew | TERMINATE |
| 6 | Optical engineer | Primary or backup secured | PAUSE |
| 9 | CT data access | ≥300 CT scans available for training | PAUSE |
| 12 | Market validation | N≥50 WTP study complete; positive signal | PIVOT REVIEW |
| 18 | CT-free algorithm | ±30mm on validation set (N≥100) | PIVOT to CT-based |

## 11.2 Termination Triggers

The project terminates with return of remaining capital if: Benchtop PoC fails at Month 3 (fundamental physics/engineering blocked); no acceptable optical engineer available by Month 9 after contingency exhausted; market validation returns <20% of projected willingness-to-pay.

# 12. Regulatory Compliance

## 12.1 EU AI Act — High-Risk Classification

HD-ECGI falls under Annex III, Category 5(b): AI systems intended to be used as safety components in medical devices.

Risk management system (Article 9): Integrated with IEC 62304 software lifecycle. Data governance (Article 10): Training data provenance, bias testing per Section 4. Transparency (Article 13): Confidence scores, SHAP values for explainability, user documentation. Human oversight (Article 14): Clinical decision support classification; human-in-loop required.

## 12.2 MDR Article 61 — Clinical Evaluation

Literature review: Existing ECGI validation studies (CardioInsight, ECVUE). Equivalence claim: Where applicable, comparison to predicate devices. Clinical investigation: Prospective validation study (Phase 2-3). Post-market surveillance: Real-world performance monitoring plan.

# Appendix A: Glossary

| Term | Definition |
| --- | --- |
| Clinically significant finding | Any detection that would change patient management pathway: STEMI, VT, WPW, complete heart block, or other conditions requiring immediate intervention |
| Marginal benefit | <10% improvement in sensitivity or specificity over 256-electrode system; not sufficient to justify additional complexity |
| Acceptable candidate | Meets minimum qualifications (PhD or 8+ years industry in optical systems); available within 60 days; salary within £90-120K band; no conflict of interest |
| Clinical decision support | Software that provides information to support clinical decisions but does not make autonomous diagnostic or treatment decisions per EU MDR Article 2(1) |
| Graceful degradation | System maintains reduced functionality under adverse conditions; for HD-ECGI: basic arrhythmia detection when electrode contact <50% |
| Strategic pivot | Fundamental change to product or market strategy; CT-free failure requires pivot to different competitive landscape |
| Willingness-to-pay (WTP) | Maximum price a customer would pay for the product; measured via Van Westendorp price sensitivity analysis |
| CCTA | Cardiac CT Angiography — imaging technique using CT with contrast agent |
| EP | Electrophysiology — study of electrical properties of the heart |
| HD-ECGI | High-Density Electrocardiographic Imaging |
| MDR | Medical Device Regulation (EU) 2017/745 |
| PACS | Picture Archiving and Communication System — medical imaging storage |
| PCR | Patient Care Record — ambulance service documentation |
| SSM | Statistical Shape Model — mathematical representation of anatomical variation |
| STEMI | ST-Elevation Myocardial Infarction — heart attack requiring urgent intervention |

# Appendix B: References

Makower J, Meer A, Denend L. FDA Impact on U.S. Medical Technology Innovation. Stanford University, 2010.

Potyagaylo D, Schulze WHW, Doessel O. A new method for reconstruction of endocardial and epicardial potentials from the body surface potential map. Biomedical Engineering, 2019.

Schulze WHW, et al. ECG imaging of ventricular tachycardia: evaluation in patients and a novel hypothesis-based reconstruction method. Clinical Research in Cardiology, 2019.

# Appendix C: Document Control

| Document Title | High-Density Electrocardiographic Imaging System |
| --- | --- |
| Subtitle | Technical Proposal & Investment Memorandum |
| GUID | HDECGI-2025-371D8NE1 |
| Internal Version | 6.0 |
| Date | 29 December 2025 |
| Author | Aaron Garcia |
| Origin Context | Technical Documentation Review |
| Contact | aaron@garcia.ltd |
| Classification | Open for comment |
| Status | Final Draft |

— END OF DOCUMENT —